Assert Crash on Resume from S4 Sleep

On a Windows 11 system using AX201 Wi-Fi NIC , a recoverable assert was observed after resuming from S4 sleep. Below is the configuration table:

POA Details:

| Platform | Generic Test Platform |
| --- | --- |
| Hardware | Generic Test NIC |
| Test | Hibernate(S4) |
| OS | Windows 11 24H2 |
| BKC | WW07.1'25(CONS) |
| Wi-Fi Driver | SBHWFW01998_23.130.0.6 |
| BT Driver | SBHBTW_PP_WEEKLY1902_23.130.25074.62572 |
| AP Name | TP-link BE1900 Tri-Band Wi-Fi 7 Router |
| AP Chip Set | Archer BE800 v1.0 |
| AP Firmware | 1.2.1 Build 20250108 rel.48985(5553) |
| AP Configuration | Dynamic EMLSR |

Logs: Resume flow logs, Driver crash logs

E-Fireman log: Fireman dump, Fireman log parser

Frequency: 7/500

Priority: P1 — Blocks test automation and causes instability in resume test cases

Steps to reproduce :
Step #1 : Flash the BKC as per configuration.
Step #2 : Install WIFI driver, then install BT, WRT2G.
Step #3 : Enable Verifier. 
Step #4 : Validate Wi-Fi state is on.
Step #5 : Connect to AP in Auto Connect mode.
Step #6 : Verify WI-FI Connection.
Step #7 :  Send DUT to Hibernate(S4) for 60 seconds.
Step #8 :  Repeat the Steps 3 to 6 for 1500 times.
Step #9 : Observe system behavior. UMAC assert is triggered on some resumes.

Logs (sample):

- Connection Flow: Includes WPA3 auth, reassoc, connect response

- Assert Log: `uCode ASSERT - UMAC, rtStatus = 0x2xxxxxxx

- Reset Flow: Triggered miniport reset, recovery completed successfully

- Frame Time: 96578.9 ms, Event: `UMAC_ERR_LOG_ASSERT`

Tools Used:

- Internal Resume Trigger Scripts

- Log Parser (equivalent to Fireman)

Note: This is a recreated demo bug report based on typical Wi-Fi test scenarios. No proprietary or confidential data is shared.